# 0195p DTAC tactile indicators and stair edgings

Branded worksection

This branded worksection Template has been developed by NATSPEC in conjunction with DTAC Pty Ltd (the Product Partner) and may be used whilst the Product Partner is licensed to distribute it. The copyright remains with NATSPEC. As with all NATSPEC worksections, it is the responsibility of the user to make sure it is completed appropriately for the project. The user should also review its applicability for local conditions and regulations. Check www.natspec.com.au for the latest updated version.

Worksection abstract

This branded worksection Template is applicable to the supply and installation of DTAC tactile indicators, edge protectors, and edging for stair nosings.

How to use this worksection

This worksection Template must be customised for each project. See A guide to NATSPEC worksections (www.natspec.com.au) for information on Template structure, word styles, and completing a worksection.

Related material located elsewhere in NATSPEC

If a listed worksection is not part of your subscription package and you wish to purchase it, contact NATSPEC.

Related material may be found in other worksections. See for example:

0315 Concrete finishes for concrete stairs and surface finishes.

0541 Access floors.

0551 Joinery for timber stairs.

0552 Metalwork – fabricated for steel stairs.

0613 Terrazzo in situ for surface finish.

0631 Ceramic tiling for tiled floors and stairs.

0632 Stone and terrazzo tiling for tiled floors and stairs.

0651 Resilient finishes for stair and floor finish.

0652 Carpets for stair and floor finish.

0654 Multilayered board flooring for stair and floor finish.

0655 Timber flooring for stair and floor finish.

0657 Resin based seamless flooring.

Documenting this and related work

You may document this and related work as follows:

Locate tactile indicators, edge protectors and stair edgings on drawings.

If using this worksection, remove tactile indicators from the other schedules as appropriate.

The Normal style text of this worksection may refer to items as being documented elsewhere in the contract documentation. Make sure they are documented.

Search acumen.architecture.com.au, the Australian Institute of Architects’ practice advisory subscription service, for notes on the following:

Guarantees and warranties.

Specifying ESD

DTAC manufacture a tactile indicator featuring a cupped underside, using less stainless steel and putting less demand on resources.

Refer to NATSPEC TECHreport TR 01 on specifying ESD.

## General

DTAC is an Australian company with over twelve years' experience in design and manufacturing excellence, all backed by industry leading support. DTAC comprises a specialist team of professionals that prides itself on offering beautiful, BCA compliant, architectural tactile ground surface indicators and stair and joinery edging products. DTAC’s range also includes urban landscape edge protection and more. DTAC’s unequalled attention to detail enables architects, designers and builders to make the right choice for aesthetic and functional conformance in every project.

### Responsibilities

#### General

Requirement: Provide DTAC tactile indicators and edging, as documented.

Documented is defined in 0171 General requirements as meaning contained in the contract documents.

### Company contacts

#### DTAC technical contacts

Website: www.dtac.com.au/contact

### Cross references

#### General

Requirement: Conform to the following:

0171 General requirements.

0171 General requirements contains umbrella requirements for all building and services worksections.

List the worksections cross referenced by this worksection. 0171 General requirements references the 018 Common requirements subgroup of worksections. It is not necessary to repeat them here. However, you may also wish to direct the contractor to other worksections where there may be work that is closely associated with this work.

NATSPEC uses generic worksection titles, whether or not there are branded equivalents. If you use a branded worksection, change the cross reference here.

### Standards

#### General

Tactile indicators: To AS/NZS 1428.4.1.

The NCC also cites AS 1428.4-1992.

Stair edging: To AS 1428.1.

The NCC cites AS 1428.1-2001 and AS 1428.1:2009.

#### Slip resistance

Classification: To AS 4586.

See NATSPEC TECHnote DES 001 on slip resistance.

### Manufacturer's documents

#### Technical manuals

DTAC Tactile ground surface indicators: www.dtac.com.au/product-category/tactile-ground-surface-indicators

DTAC Stair & tread edging: www.dtac.com.au/product-category/stair-treads-edging

DTAC Edge protector: www.dtac.com.au/product-category/urban-products

### Submissions

#### Products and materials

Type tests: Submit results, as follows:

Slip resistance of tactile indicators and edgings.

The DTAC range of tactile indicators and edging are tested to a range of slip resistance tests including AS 4586 Appendices A, C & D. Refer to the DTAC product Technical Data sheets for product specific test results.

Luminance contrast testing: Submit evidence of conformity to AS/NZS 1428.4.1 Appendix E and AS 1428.1 Appendix B.

Product luminance test results are included on the DTAC technical data sheets which are presented by product type.

#### Prototypes

General: Provide a prototype of the DTAC product installed in the finished substrate.

Location and extent:

Consider adding the following Optional style text.

Incorporation: Subject to approval, incorporate the prototype in the completed works.

Preferably show the location and extent on the drawings. Delete if a prototype is not required.

#### Warranties

Tactile indicators, edge protectors and stair edging: Submit DTAC product and installation warranties.

### Inspection

#### Notice

Inspection: Give notice so that inspection may be made of the completed substrate ready for tactile indicators and edging installation.

A site inspection can be conducted before installation.

Amend to suit the project, adding critical stage inspections required.

Hold points, if required, should be inserted here.

## Products

### General

#### Product substitution

Other products: Conform to SUBSTITUTIONS in 0171 General requirements.

SUBSTITUTIONS in 0171 General requirements sets out the submissions required if the contractor proposes alternative products. Refer also to NATSPEC TECHnote GEN 006 for more information on proprietary specification.

### DTAC tactile indicators, edge protectors and stair edging

DTAC can provide technical advice so that the appropriate tactile and edging products are specified.

#### Warning tactile products

Classic: Tactile indicators with a concentric circle design machined or injection moulded on the horizontal face and a smooth outer edge.

Available in type 316 stainless steel or solid brass, or in black, yellow or light grey UV stabilised thermoplastic urethane (TPU).

Terraced: Tactile indicators with a concentric circle design machined or injection moulded on the horizontal face and a terraced slip-resistant outer edge.

Available in type 316 stainless steel, solid brass, or type 316 stainless steel outer edge with a black UV stabilised thermoplastic urethane (TPU) top.

Classic Colourmax: Tactile indicators with a concentric circle design machined on the type 316 stainless steel horizontal face with a smooth injection moulded UV stabilised thermoplastic urethane (TPU) outer edge.

Available in black or yellow.

Handrail button: Machined domed type 316 stainless steel button.

EcotacTM Classic: Tactile indicators with a cupped underside, a concentric circle design machined on the type 316 stainless steel horizontal face and a smooth outer edge.

EcotacTM Terraced: Tactile indicators with a cupped underside, a concentric circle design machined on the type 316 stainless steel horizontal face and a terraced slip-resistant outer edge.

UltimatTM Tactile Classic: UV stabilised thermoplastic urethane (TPU) injection moulded tactile with Classic tactile indicators on the horizontal face.

Available in black, yellow, light grey or ivory.

#### Directional tactile products

EcotacTM Classic: Tactile indicator type 316 stainless steel with a cupped underside, a grooved design machined on the horizontal face and a smooth outer edge.

EcotacTM Terraced: Tactile indicator type 316 stainless steel with a cupped underside, a grooved design machined on the horizontal face and a terraced slip-resistant outer edge.

Classic: UV stabilised thermoplastic urethane (TPU) injection moulded tactile with an undulating groove design on the horizontal face and a smooth outer edge.

Available in black, yellow or light grey.

#### Integrated tactile products

Classic: Type 316 stainless steel plate with Classic tactile indicators integrated on the horizontal face.

Classic Black Top: Type 316 stainless steel plate with Classic Black tactile indicators integrated on the horizontal face.

#### Edge protector

Round button: 35 mm diameter urban edge protector.

Available in type 316 stainless steel or solid brass.

Bevelled button: 35 mm diameter urban edge protector.

Available in type 316 stainless steel or solid brass.

Thin bar: 5 mm thick type 316 stainless steel bar.

Thick bar: 10 mm thick type 316 stainless steel bar.

#### Stair edging products

Corduroy edging: Extruded aluminium sections with a continuous corduroy pattern.

Anodised finishes available include natural, black or gold. Sections available flat or angled.

Pinstripe edging: Extruded smooth, blunt edged aluminium sections with an intermittent high and low profile.

Anodised finishes available include natural or black, or a combination of both black and natural. Sections available flat or angled.

Pleat edging: Extruded smooth, aluminium sections with an intermittent continuous corduroy pattern.

Anodised finishes available include natural or black. Sections available flat or angled.

Stitch edging: Extruded aluminium sections with a continuous stitch pattern.

Anodised finishes available include natural or black, or a combination of both black and natural. Sections available flat or angled.

Suede edging: Extruded anodised aluminium sections with a continuous band of silicon carbide.

Available in, in either natural or black. Sections available flat or angled.

Urban edging: Continuous band of silicon carbide inserted into anodised aluminium extrusions.

Available in flat, angle or carpet profiles in natural or black anodised aluminium. Suitable for all stable finished outdoor or indoor surfaces and substrates.

Urban edging - Mill finished aluminium: Intermittent striations of silicon carbide inserted into aluminium extrusions.

Available in flat or angle profiles. Suitable for all stable finished outdoor or indoor surfaces and substrates.

RuggedTM edging: Continuous band of fibre glass reinforced plastic extrusion.

Available in angled profiles, in yellow or black.

Up to 40 µm thick, the DTAC hard anodising process produces a tough protective coating of aluminium oxide on the surface of the aluminium. This results in a uniform, hard wearing, scratch resistant surface ideal for commercial applications.

## Execution

### General

#### Substrate preparation and installation of DTAC products

Requirement: To DTAC’s recommendations and fitting instructions.

Location: As documented.

Classic, Terraced and Classic Colourmax:

Stone, masonry, timber or vinyl substrates: Drill and pressure fit.

Vitrified porcelain, ceramics, glass or metal substrates:

Select Diamond core/pressure fit or Direct stick from the options in the DTAC technical data sheet,

Carpet or carpet tile substrates: Drill and screw.

Handrail button: Drill and glue to substrate.

UltimatTM Tactile: Direct stick to substrate with DTAC anchor adhesive.

Button and bar urban edge protector:

Stone, masonry, timber or vinyl substrates: Drill and glue.

Vitrified porcelain, ceramics, glass or metal substrates: Diamond core drill and glue.

#### Fixing stair edging

Corduroy, Pinstripe, Pleat, Stitch, Suede:

Stone, masonry, timber or vinyl substrates: Adhesive fix.

Vitrified porcelain, ceramics, glass or metal substrates: Adhesive fix.

Carpet or carpet tile substrates: Screw-fix or adhesive fix.

Urban and RuggedTM: Screw-fix with optional adhesive.

### Testing

#### Completion tests

Slip resistance of completed installation: To AS 4663.

Delete if not required. See NATSPEC TECHnote DES 001. The wet-barefoot inclining platform test and the oil-wet inclining platform test cannot be performed in situ.

Luminance contrast testing of completed installation: Submit evidence of conformity to AS/NZS 1428.4.1 Appendix E and AS 1428.1 Appendix B.

DTAC can test substrates for luminance contrast to make sure the installed DTAC tactile indicators and edging conform to AS/NZS 1428.4.1 Appendix E for tactile indicators and AS 1428.1 Appendix B for edging. Delete if not required.

### Completion

#### Warranties

Conditions: Installation by DTAC or DTAC approved by installer.

Warranty period: 2 years.

The DTAC range of tactile indicators and edging are guaranteed for 2 years against defects in materials and workmanship, provided the products are fitted and inspected in accordance with the manufacturer’s specifications.

## Selections

Schedules are a tool to specify properties required for products or systems. If the principal permits documentation of the product or system by proprietary name, some of the properties may be unnecessary and can be deleted. Document the product or system's location here and/or on the drawings with a matching product code. Refer to NATSPEC TECHnote GEN 024 for guidance on using and editing schedules.

### DTAC products

#### Tactile indicator and edge protector schedule

|  | TI1 | TI2 | TI3 |
| --- | --- | --- | --- |
| Type |  |  |  |
| Product code |  |  |  |
| Design |  |  |  |
| Material/Colour |  |  |  |
| Slip-resistance classification |  |  |  |
| Substrate |  |  |  |

The codes in the header row of the schedule designate each application or location of the item scheduled. Edit the codes to match those in other contract documents.

Type: Select from Warning tactile, Directional tactile, Integrated tactile or Urban edge protector.

Product code: Refer to the DTAC technical data sheets.

Design: Select from:

Warning tactile indicator: Classic, Terraced, Classic Colourmax or Handrail button.

Directional tactile indicator: EcotacTM Classic, EcotacTM Terraced or UltimatTM Tactile Classic.

Integrated tactile indicator: Classic or Classic Black Top.

Edge protector: Button bevelled, Button round, Bar thin or Bar thick.

Material/Colour: A colour contrast may be required, in both wet and dry conditions, between the tactile indicators and the adjacent surface. The colour is to provide a luminance contrast to the surrounding surface to AS/NZS 1428.4.1 Appendix E.

Material/Colour of warning tactile indicators: Select from:

Classic: Stainless steel, Brass, or Black, Yellow or Light grey urethane.

Terraced: Stainless steel, Brass, or Stainless steel outer edge with a black urethane top.

Classic Colourmax: Black or Yellow.

Handrail button: Stainless steel.

EcotacTM Classic: Stainless steel.

EcotacTM Terraced: Stainless steel.

UltimatTM Tactile Classic: Black, Yellow, Light grey or Ivory urethane.

Material/Colour of integrated tactile products: Select from:

Classic: Stainless steel.

Classic Black Top: Stainless steel with black tactile.

Material/Colour of edge protector: Select from:

Button: Stainless steel or Brass.

Bar: Stainless steel.

Slip resistance classification: Refer to NATSPEC TECHnote DES 001, SA HB 197 and SA HB 198. Select the slip resistance test and classification to suit the location and application.

Substrate: Nominate the material of the substrate for fixing the tactile indicator.

#### Stair edging schedule

|  | SE1 | SE2 | SE3 |
| --- | --- | --- | --- |
| Product code |  |  |  |
| Application |  |  |  |
| Design |  |  |  |
| Colour |  |  |  |
| Slip-resistance classification |  |  |  |
| Substrate |  |  |  |

The codes in the header row of the schedule designate each application or location of the item scheduled. Edit the codes to match those in other contract documents.

Product code: Refer to the DTAC technical data sheets.

Application: e.g. Stair nosing to BCA requirements, or Edging to finishes as detailed.

Design: Select from:

Corduroy.

Pinstripe.

Pleat.

Stitch.

Suede.

Urban.

Urban - Mill finished aluminium.

RuggedTM.

Colour: Select from:

Corduroy edging: Natural, Black or Gold.

Pinstripe edging: Natural, Black, or Black brushed.

Pleat edging: Natural or Black.

Stitch edging: Natural, Black, or Black and natural.

Suede edging: Natural or Black.

Urban edging: Natural or Black anodised aluminium.

RuggedTM edging: Yellow or Black.

Slip resistance classification: Refer to NATSPEC TECHnote DES 001, SA HB 197 and SA HB 198. Select the slip resistance test and classification to suit the location and application.

Substrate: Nominate the material of the substrate for fixing the stair edging.

REFERENCED DOCUMENTS

The following documents are incorporated into this worksection by reference:

AS 1428		Design for access and mobility

AS 1428.1	2021	General requirements for access - New building work

AS/NZS 1428.4.1	2009	Means to assist the orientation of people with vision impairment - Tactile ground surface indicators

AS 4586	2013	Slip resistance classification of new pedestrian surface materials

AS 4663	2013	Slip resistance measurement of existing pedestrian surfaces

The following documents are mentioned only in the Guidance text:

AS 1428		Design for access and mobility

AS 1428.1	2001	General requirements for access - New building work

AS 1428.4	1992	Tactile ground surface indicators for the orientation of people with vision impairment

SA HB 197	1999	An introductory guide to the slip resistance of pedestrian surface materials

SA HB 198	2014	Guide to the specification and testing of slip resistance of pedestrian surfaces

NATSPEC DES 001	2018	Slip resistance performance

NATSPEC GEN 006	2015	Product specifying and substitution

NATSPEC GEN 024	2021	Using NATSPEC selections schedules

NATSPEC TR 01	2021	Specifying ESD